Bit Full Adder

1. A 1-Bit full adder circuit is used to add 2 single bits according to the rules of standard binary addition.

2. The circuit has 3 inputs, two of which are the bits being added (A and B) and a carry input (Cin).

3. The carry input is the bit that is carried over from a previous addition operation, or is zero otherwise.

4. The circuit produces 2 outputs which are bits representing sum (S) and carry (Cout).

5. The logic for addition is- A XOR B XOR Cin. It can be derived by simplifying the truth table using K-map.

6. The logic for determining the carry output is- ((A XOR B)*Cin) + (A*B) which can again be derived from the following truth table.

| A | B | Cin | S | Cout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |